Table 1 Linearities, intra- and interday precisions and accuracies, limits of detection (LOD), limits of quantification (LOQ) of GHBA

est-Me-GHBA, GBHA-2TMS, cyclo-GHBA/GBL.

| Tested parameter | Sample matrix |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DCM solution |  |  |  | Plasma |  |  |  |
|  | Compound |  |  |  |  |  |  |  |
|  | GHBA | est-Me-GHBA | GBHA-2TMS | Cyclo-GHBA/GBL | GHBA | est-Me-GHBA | GBHA-2TMS | Cyclo-GHBA/GBL |
| Linearity – calibration level for the low range (R2) | 0.9994 | 0.9985 | 0.9982 | 0.9990 | 0.9984 | 0.9971 | 0.9968 | 0.9983 |
| Linearity - calibration level for the high range (R2) | 0.9996 | 0.9991 | 0.9987 | 0.9994 | 0.9991 | 0.9979 | 0.9977 | 0.9992 |
| Intraday precision (% RSD) | 2.99 | 3.38 | 3.57 | 3.05 | 3.31 | 3.74 | 3.93 | 3.35 |
| Interday precision (% RSD) | 3.62 | 4.01 | 4.37 | 3.72 | 3.98 | 4.47 | 4.83 | 4.11 |
| Intraday accuracy (%) | 99.09 | 98.12 | 97.92 | 98.97 | 94.72 | 92.81 | 92.58 | 93.57 |
| Interday accuracy (%) | 97.96 | 97.21 | 96.85 | 97.63 | 92.58 | 91.94 | 91.54 | 92.22 |
| LOD (µg/mL) | 0.22 | 0.17 | 0.13 | 0.04 | 0.28 | 0.21 | 0.13 | 0.04 |
| LOQ (µg/mL) | 0.75 | 0.56 | 0.42 | 0.13 | 0.94 | 0.70 | 0.46 | 0.13 |

R2 coefficient of determination, RSD relative standard deviation, LOD limit of detection, LOQ limit of quantification