Supplemental Tables

Supplemental Table I. List of cannabinoids and their abbreviations analyzed for effect of MAP on storage degradation in cannabis.  Cannabidiol and cannabicycloic acid were also analyzed but were below non-detectable limits.

| Compound | Abbreviation | CAS Registration Number |
| --- | --- | --- |
| Δ9-Tetrahydrocannabinol | Δ9-THC | 1972-08-3 |
| Δ8-Tetrahydrocannabinol | Δ8-THC | 5957-75-5 |
| Cannabichromene | CBC | 20675-51-8 |
| Cannabichromenic acid | CBCA | 20408-52-0 |
| Cannabidiolic acid | CBDA | 1244-58-2 |
| Cannabidivarin | CBDV | 24274-48-4 |
| Cannabidivarinic acid | CBDVA | 31932-13-5 |
| Cannabigerol | CBG | 25654-31-3 |
| Cannabigerolic acid | CBGA | 25555-57-1 |
| Cannabicyclol | CBL | 21366-63-2 |
| Cannabinol | CBN | 521-35-7 |
| Cannabinolic acid | CBNA | 2808-39-1 |
| Tetrahydrocannabinolic acid | THCA | 23978-85-0 |
| Tetrahydrocannabidivarin | THCV | 28172-17-0 |
| Tetrahydrocannabidivarinic acid | THCVA | 39986-26-0 |

Supplemental Table II. List of terpenoids analyzed for effect of MAP on storage degradation in cannabis.  An additional 38 terpenoids were tested but were below non-detectable limits.

| Terpene Name | Terpene Name |
| --- | --- |
| 𝛼-Pinene | Fenchone |
| Terpinolene | cis-𝛽-Ocimene |
| Camphene | trans-𝛽-Ocimene |
| Linalool | Caryophyllene oxide |
| 𝛽-Pinene | Octyl acetate |
| 𝛽-Myrcene | Borneol |
| 𝛽-Caryophyllene | 𝛼-Terpineol |
| 𝛼-Humulene | 𝛼-Bisabolol |
| Limonene | Terpinen-4-ol |
| trans-Nerolidol | Geranyl acetate |
| Eucalyptol | 𝛽-Cedrene |
| Guaiol |  |

Supplemental Table III. Vanquish HPLC parameters for analysis of cannabinoids.

| Column | Ascentis Express 90 Å C18 15 cm x 2.1 mm x 2 µm |
| --- | --- |
| Column Temp | 30 °C |
| Mobile Phase A | 5 mM ammonium formate in water + 0.1% v/v formic acid |
| Mobile Phase B | 0.1% formic acid in acetonitrile |
| Flow Rate | 0.4 mL/min |
| Gradient | 0.0 min: 70% B 3.0 min: 90% B 5.0 min: 90% B 5.1 min: 98% B 6.0 min: 98% B 6.1 min: 70% B 8.0 min: 70% B |
| Detection | UV 228 nm |
| Injection | 25 μL |
| Elution Order | Ibuprofen, CBDVA, CBDV, CBDA, CBGA, CBG, CBD, THCV, THCVA, CBN, CBNA, Δ9-THC, Δ8-THC, CBL, CBC, THCA, CBCA, CBLA |

Supplemental Table IV. GC and injector parameters for analysis of terpenoids.

| Injection Volume (µL) | 1 |
| --- | --- |
| Liner | 4 mm Splitless Single Taper |
| Inlet (°C) | 220 |
| Inlet Module and Mode | SSL, Split |
| Carrier Gas (mL/min) | Helium (1.5) |
| Split Flow (mL/min) | 6 |
| Oven Temperature Program: |  |
| Temperature 1 (°C) | 60 |
| Hold Time (min) | 0.5 |
| Temperature 2 (°C) | 130 |
| Rate (°C/min) | 50 |
| Hold Time (min) | 3 |
| Temperature 3 (°C) | 140 |
| Rate (°C/min) | 5 |
| Hold Time (min) | 0 |
| Temperature 4 (°C) | 250 |
| Rate (°C/min) | 22 |
| Hold Time (min) | 0 |
| Temperature 5 (°C) | 300 |
| Rate (°C/min) | 120 |
| Hold Time (min) | 2 |

Supplemental Table V. ISQ 7000 mass spectrometer parameters for analysis of terpenoids.

| Transfer Line (°C) | 250 |
| --- | --- |
| Ionization Type | EI |
| Ion Source (°C) | 300 |
| Acquisition Mode | SIM |