This MATLAB program estimates the FRET efficiency in the wing disc at each Z-plane from a sensitized emission method of FRET.

Clearing memory and Command terminal

clc;

clear;

Selecting input the directory and file path

read_folder = '-Add folder path here-';

cd(read_folder);

YFP_files = dir('C2*.tif');                              Pattern for YFP files

CFP_files = dir('C1*.tif');                              Pattern for CFP files

Drawing ROIs for all the discs

F = 2;                                                   Enter the number of Discs

back_rect_array = [];                                    Initialize background rectangle array

mask_array = [];                                         Initialize mask array

file_inf = imfinfo(CFP_files(1).name);

N = numel(file_inf); %                                   Estimate the number of Z slices

[row,col] = size(CFP_files(1).name);                     Estimate image size

for d=1:F

test = imread(CFP_files(d).name,1);

[row,col] = size(test);

CFP = uint16(zeros(row,col,N));YFP = uint16(zeros(row,col,N));

for i=1:N

CFP(:,:,i) = imread(CFP_files(d).name,i);    Generate 3-D array for CFP

YFP(:,:,i) = imread(YFP_files(d).name,i);    Generate 3-D array for YFP

end

YFP_max = mean(YFP,3);                           Generate mean projected image for YFP

N = numel(CFP(1,1,:));

H = fspecial('average',[5,5]);                   Creating an averaging kernel

[I_crop,back_rect] = imcrop(YFP_max,[]);         Get background rectangle

imshow(YFP_max,[]);

h = imfreehand();  %                             Draw a freehand ROI

mask = h.createMask; %                           Convert freehand ROI into mask

back_rect_array = cat(1,back_rect_array,back_rect); Generate array of Background ROI

mask_array = cat(3,mask_array,mask);             Generate an array of masks

end

Computing FRET the wing discs in 3-Dimension for all the discs selected

All_disc_mean = [];

count =1;

for d=1:F

for i=1:N;

CFP(:,:,i) = imread(CFP_files(d).name,i);     Generate a 3-D array for CFP

YFP(:,:,i) = imread(YFP_files(d).name,i);     Generate a 3-D array for YFP

end

H = fspecial('average',[5,5]);                Generate a 5X5 smoothing kernel

back_rect1 = back_rect_array(count,:);        Select a background ROI from array

mask1 = mask_array(:,:,count);                Select a mask from array of masks

FRET_Z = zeros(row,col,N);Tot_donor_Z = zeros(row,col,N);

for j=1:N

back_cfp = imcrop(CFP(:,:,j),back_rect1); Generate background image for CFP

back_yfp = imcrop(YFP(:,:,j),back_rect1); Generate background image for YFP

mn1 = mean(mean(back_cfp));               Compute mean background for CFP

mn2 = mean(mean(back_yfp));               Compute mean background for YFP

cfp_filt = imfilter(CFP(:,:,j),H); %.     5X5 smoothing of the image for CFP

yfp_filt = imfilter(YFP(:,:,j),H); %.     5X5 smoothing of the image for CFP

cfp_back_sub = double(cfp_filt - mn1);    Subtract background from CFP image

yfp_back_sub = double(yfp_filt - mn2);    Subtract background from CFP image

leak = 0.4*cfp_back_sub;                  Computing leakage

PFRET = yfp_back_sub - leak;              Get FRET intensity

tot_donor = PFRET + cfp_back_sub;         Get Total donor intensity

FRET = PFRET./tot_donor;                  Estimate FRET Efficiency

FRET_Z(:,:,j) = FRET;

Tot_donor_Z(:,:,j) = tot_donor.*mask1;    Get masked total donor intensity

end

Tot_don_lin = Tot_donor_Z(Tot_donor_Z>0);

mn = mean(Tot_don_lin);                       Get mean intensity of Total Donor

Generate background by rolling ball method

tot_don_back = imopen(Tot_donor_Z,strel('disk',25));

Tot_don_backsub = Tot_donor_Z - tot_don_back; Subtract Background

Tot_don_filt = medfilt3(Tot_don_backsub);

Tot_don_thresh = (Tot_don_filt>0.20*mn);      Threshold the total donor

Thresholding FRET signal

FRET_thresh = zeros(row,col,N);

for k=1:N

FRET_thresh(:,:,k) = FRET_Z(:,:,k).*Tot_don_thresh(:,:,k);

end

Saving FRET values as a MATLAB matrix

filename = strcat('ATP_','Disc_',num2str(d,'%03d'),'.mat');

save(filename, 'FRET_thresh','Tot_donor_Z');      Save FRET and total donor matrix

count = count+1

FRET_lin = FRET_thresh(FRET_thresh>0);

mn_FRET = mean(FRET_lin);

All_disc_mean = cat(1,All_disc_mean,mn_FRET);

end