Supplementary materials

Table S1: LC gradient settings. TC = trapping column, AC = analytical column, MS = mass spectrometer

| Time (min) | Pumps 1 and 2: analytical column (AC) |  | Pumps 3 and 4, Pump 5: trapping column (TC) |  |  | Switching valve |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mobile Phase B (%) | Flow rate (µL/min) | Mobile Phase B (%) | Flow rate (µL/min) | Flow rate Pump 5 (µL/min) |  |
| 0.00 | 30 | 300 | 50 | 500 | 300 | TCwaste ACMS (load) |
| 1.00 |  |  |  |  |  | TCACMS (elute) |
| 1.20 | ↓ (linear gradient) |  |  |  |  |  |
| 1.30 |  |  |  | 20 | 20 |  |
| 7.50 | 97.5 |  |  |  |  | TCwaste, ASMS (re-equilibration) |
| 7.60 |  |  | 100 | 1000 |  |  |
| 10.00 |  |  |  |  |  |  |
| 10.10 | 30 |  | 50 | 500 | 300 |  |
| 12.00 |  |  |  |  |  | TCwaste ACMS |

Table S2: MRM transitions and MS settings for THC and THC-COOH in DBS. DP=Declustering potential, EP=entrance potential, CE=collision energy, CXP=collision cell exit potential.

| Analyte | Q1 / Da | Q3 / Da | DP / V | EP / V | CE / V | CXP / V |
| --- | --- | --- | --- | --- | --- | --- |
| THC MRM1 | 315.200 | 193.100 | 30 | 10 | 30 | 15 |
| THC MRM2 | 315.200 | 123.000 | 30 | 10 | 42 | 15 |
| THC-COOH MRM1 | 345.200 | 327.143 | 30 | 10 | 21 | 22 |
| THC-COOH MRM2 | 345.200 | 299.156 | 30 | 10 | 26 | 24 |
| THC-COOH MRM3 | 345.200 | 193.126 | 30 | 10 | 36 | 13 |
| THC-D3 MRM1 | 318.280 | 196.300 | 30 | 10 | 29 | 4 |
| THC-COOH-D3 MRM1 | 348.240 | 330.300 | 30 | 10 | 19 | 4 |